Zhou Lab Protocol for Microarraying and Fluorescent Labeling of Genomic DNA

# Microarraying Notes:

- Parts 1-4 can be done ahead of time in batches and stored dry at room temperature.

- Carry slides around in a sealed box.

- Prepare 1M boric acid pH 8.0 (with NaOH) ahead of time.  Store at room temperature (RT).

- Place a stirheat plate in the hood.

- Prepare a water bath (beaker with > 500 ml) at 95°C with double distilled millipore water ahead of time.

- Mark slides with position of array and number at the array free area before rehydration step using a diamond-tip glass marker.

##### Printing Slides

##### See Liyou

##### II.  	Rehydration and Snap Drying

- Hold the slides, array-side down, over a hot water bath (50-65°C) until the spots are glistening (about 1-5 seconds).

- Immediately snap-dry the slides array-side up on an inverted hot heating block at 80°C for 5 seconds.

## III. 	Crosslinking

- Crosslink the DNA in a Stratagene UV 1800 Stratalinker at 65 mJ energy.

## IV.	Blocking (Omitted for SuperAmine And SuperAlddehyle slides)

- Clean a glass slide staining dish and slide rack with 95% ethanol. (Only use a glass staining dish and rack since metal will react with the chemicals used for this process).

- Add 3.58 g of succinic anhydride (Sigma Chemical Co.) to the staining dish.

- In a fume hood, add 200 ml of 1-methyl-2-pyrrolidonone (Sigma Chemical Co.).

- Stir at high speed to dissolve the boric acid.  Then add 8.95 ml of 1 M boric acid (pH 8.0).

- Place the slides in the glass slide rack and immediately (WITHIN 2 SECONDS) dunk the slides in the glass staining dish 5 times up and down in the blocking solution.  Then cover the staining dish and stir slowly for 15 minutes.

V. 	Denaturation

- Move the 95°C water bath to the hood.

- Immediately dunk the slides 3 times up and down in the water bath and incubate for 2 minutes.

- Immediately dunk the slides 5 times up and down in 95% ethanol.

- Dry the slides using compressed air.

- Store the dried slides in a slide box at room temperature until ready for use.

VI.  	Protocol for SYBR Green DNA Staining to Check DNA Retention on Slide

- Thaw the SYBR Green stain in the dark and make a 1:10,000 dilution.

- Place a cover slip on the slide over the array spots.

- Add 35 µl of the diluted stain to the slide and allow the stain to move under the cover slip by capillary action.

- Fill the bottom of a tip box with warm water to act as a humidity chamber.

- Place the slide on the tip rack and close the cover.

- Place the box in a drawer or other dark area.

- Incubate the slide for ½ to 1 hour at room temperature.

- Rinse the slide several times with fresh milli-Q water.

- Either air dry or spin dry the slide.

- Scan the slide using the Cy3 channel (green laser) to check the array quality.

## Genomic DNA Labeling with Cy3- or Cy5- dCTP

|  | ### 1 Rx | ### 10 Rx |
| --- | --- | --- |
| sterile dH2O | 29.2 µl | 34 µl |
| 3µg/µl random 6-mers | 0.5 µl | 5 µl |
| 1µg/µl genomic DNA | 2.5 µl | 25 µl |

- Thoroughly mix ingredients together and briefly centrifuge.  Boil for 2 minutes (95°C

- for 5 minutes in a PCR thermocycler).  Immediately chill on ice.

2.   Prepare Rx mix:

|  | ### 1 Rx | ### 10 Rx |
| --- | --- | --- |
| 10 Buffer [NEB] | 4 µl | 10 µl |
| 5mM each dATP, dTTP, and GTP, 2.5mM dCTP | 0.4 µl | 3 µl |
| 1 mM dCTP | 0.4 µl | 3 µl |
| 0.1 M DTT | 1 µl | 10 µl |
| 4.7-5 U/µl large Klenow Fragment | 2 µl | 10 µl |

- 3.   Mix and incubate in the dark at 37°C for 2 hours.

- 4.   Boil 3 minutes and cool down on ice.

## VIII.	Purify the Probe using a Qiagen PCR Purification Column

- Add 500 µl of PB Buffer and mix the probe (mix the solutions by pipetting up and down).  Centrifuge at 12,000  g for 30 seconds.

- Wash the column with 500 µl of PE Buffer twice and centrifuge at 12,000  g for 30 seconds.

- Elute the labeled probe with 50 µl of 10 mM Tris-HCl pH 8.5 by incubating in a

- bucket with lid covering it for 2 minutes, and centrifuging at 12,000 x g for 1 min.

## IX. 	Concentrate the Probe using a SpeedVac

## Speedvac the eluted probe at 43°C for about90 minutes until about 5 ul in final

## volume remains.

## X. 	Hybridization

The following is the protocol for hybridization solution volumes of 12 µl for a 22 x 22 cover slip or 30 µl for a 24 x 50 mm cover slip.

- Heat a water bath to 65°.

- Clean the cover slips by rinsing sequentially in 50-ml-beakers containing (a) 95% ethanol, (b) 0.2% SDS, and (c) deionized H2O.

- Then rinse the slides under a running stream of fresh milli-Q water.  Dry the slides by standing up or with compressed air.

- Dissolve or dilute the probe in final 10µl of sterile distilled water. Mix the following:

|  |  |  | Final Concentration |
| --- | --- | --- | --- |
| Cy probe or mix | 20.56 µl | 9 µl |  |
| 20 SSC | 5.04 µl | 2.1 µl | 3.25 SSC |
| Herring Sperm DNA [10 µg/µl] | 2.4 µl | 1 µl | 0.77 µg/µl |
| 5% SDS | 2.0 µl | 0.8 µl | 0.31% |
| Total Volume | 30 µl | 12.9 µl |  |

- Boil for 2 minutes (or 95°C for 5 minutes) and spin down for 2 minutes at room temperature.

- Blow slides and cover slips with bottled air to remove dust.

- Pipet 30 µl of the probe/ hybridization mix onto the cover slip, avoiding bubble formation. Let the moisture wick under the surface of the cover slip

- Place the slide into an Array IT hybridization chamber.

- Pipet 3 SSC into the hydration wells on either side of the slide.

- Close the hybridization chamber and make sure that a tight seal is formed.

- Incubate in the dark in a 65°C water bath for 5-12 hours or over night.

## Washing

- Prepare the following solutions (see the table below).  Gently dip the slides into solution I to remove the cover slip.

- Cover each beaker with an inverted ice bucket and place on a gently rotating shaker.  Sequentially wash the slides in each solution for the time stated in the table below.

|  | 1 SSC/0.2% SDS | 0.1 SSC/0.2% SDS | 0.1 SSC |
| --- | --- | --- | --- |
| Deionized Water | 470 ml | 493 ml | 497.5 ml |
| 20 X SSC | 25 ml | 2.5 ml | 2.5 ml |
| 20% SDS | 5 ml | 5 ml | 0 ml |
| Washing Time | 5 min | 5 min | 30 sec |

3. Dry the slides using compressed air. The slides are ready for scanning.  Store the slides in the dark.  The signal for Cy5 will begin to fade after a week.